THIS IS IMPORTANT INFORMATION FOR POCKET ADDIO MODULES

********************************************************

HARDWARE NOTICE:

WHEN OPERATING THE POCKET AD BOX

1. FIRST CONNECT THE POCKET AD BOX THRU THE PRINTER PORT.

2. SECONDLY, INPUT THE POWER OF THE POCKET AD BOX.

WARNING:  DO NOT INPUT THE POWER FIRST, OTHERWISE IT WILL  DAMAGE

THE AD CHIP.

SOFTWARE NOTICE:

IF YOU WISH TO USE A LONG CABLE MORE THAN 30CM. FOR THE POCKET AD

THRU  THE  PRINTER  PORT, YOU NEED TO MAKE SOME  CHANGES  IN  THE

SOURCE  PROGRAM  WHICH WE PROVIDE IN THE MANUAL.  SEE  CHAPTER  4

PAGES 10-21.

YOU  NEED TO PROVIDE MORE DELAY IN UDELAY FUNCTION.  (ALL  UDELAY

FUNCTION IN THE PROGRAM)

EG.

IN THE ORIGINAL PROGRAM (USING NORMAL CABLE)

outport (DATA(port_addr),x|CLK);

udelay (1);

REVISED PROGRAM (USING LONG CABLE)

outportb (DATA(port_addr),x|CLK);

udelay (5);

NOTE: IF YOU USE MORE THAN 1 METER, YOU NEED TO ADD MORE DELAY.

date: Jan. 4, 1996

********************************************************

DCS (DATA CAPTURE SOFTWARE) - UPDATE

We  have our standard DCS software package for both  AD/DA  cards

and  POCKET  AD modules. The following products can use  our  DCS

program:

1.  12 bit AD/DA card

2.  Super 12 bit AD/DA card

3.  Super 14 bit AD/DA card

4.  High Performance AD/DA card

5.  Pocket AD modules

a.  10 bit Pocket AD

b.  12 bit Pocket AD

For  the operating procedure of our DCS program, please refer  to

our DCS Software Manual. The following additional features in the

DCS program are:

1. F1 - Operation is still be the same

2. F2 -   This  hardware config can be set to AD/DA card  or

Pocket AD modules.

If  you choose to use the Pocket  AD  modules

you  need not to answer the ADDRESS  Mapping,

CONVERT  Loop  Count.  The  other   parameter

setting would be as follows:

a. Resolution - either 10bit or 12bit

b. Total Channels - 11 channel maximum

c. Port  Address -  the  corresponding  LPTn

address

d. BOX ID - the set BOX ID of the Pocket AD

3. F3 - Operation is still be the same

4. F4 - Operation is still be the same

5. F5 - Operation is still be the same

6. F6 - Operation is still be the same

7. F7 - Operation is still be the same

8. F8 - Operation is still be the same

9. F9 - Operation is still be the same

10. F10- Operation is still be the same

For  more  detailed usage and function, please refer to  the  DCS

user manual.

******************* end of file ************************

####################