How to use POCKET.EXE (POCKET ADDIO Test Program)

Before  running the test program, please connect the  analog

signals to Pocket A/D channels and connect the digital signals to

the  pocket DIO channel, do not input the voltage out  of  range.

The functions of test program are specified below:

[L]:LPT         Select print port address "LPT1(3BC),  LPT2(378),

LPT3(278)"

[+,-]:BOXID     Specify box id (address) which be set by hardware

switch, press "+" or "-" to select box address.

[B]:BOXNO       Create new box, maximum five boxes

[A]:A/D         Specify box is A/D input

[R]:RESOLUTION  Specify A/D input resolution (10 bit or 12 bit)

[V]:VOLTAAGE    Select A/D input range (0 - 5 V or 0 - 10 V)

[D]:DIO         Specify box is digital input and output

[0-9]:TOGGLE OUTPUT VALUE

Test relay Output Control,"1" relay channel 1,"2"

relay   chaannel 2, ... "8" relay channel 8,  "0"

all relay channel ON, "9" all relay channel OFF

[G]:GO          Start testing

[S]:STOP        Stop testing

[Q]:QUIT        Quit the program

date: Dec. 25, 1995

updated Feb. 28, 1996#############################################################